Day – 4 : Assi

List 3 key differences between Ethernet cables and fiber optic cables, and give one real-world example where each type would be used (for example, home WiFi router, data center, college campus backbone).

Ans :

| Feature | Ethernet Cable | Fiber Optic Cable |
| --- | --- | --- |
| Transmission Medium | Uses electrical signals through copper wires. | Uses light signals through glass or plastic fibers. |
| Speed & Distance | Suitable for short distances (up to about 100 meters) with speeds like 1 Gbps or 10 Gbps. | Supports much higher speeds and can transmit data over many kilometers with very little signal loss. |
| Cost & Durability | Less expensive, easy to install, but more affected by electrical interference (EMI). | More expensive, harder to install, but immune to electrical interference and more reliable over long distances. |

Draw and label a diagram showing the internal wiring of a straight-through Ethernet cable (use T568B wiring standard), indicating the color of each wire and its pin number.

Ans :

RJ-45 Connector (Front View - Clip Facing Away)

+----+-----------------+------------------+

|Pin | Wire Color      | T568B Standard   |

+----+-----------------+------------------+

| 1  | White/Orange    | TX+              |

| 2  | Orange          | TX−              |

| 3  | White/Green     | RX+              |

| 4  | Blue            |                  |

| 5  | White/Blue      |                  |

| 6  | Green           | RX−              |

| 7  | White/Brown     |                  |

| 8  | Brown           |                  |

+----+-----------------+------------------+

End A (T568B)                    End B (T568B)

Pin 1 ─────────────────────────► Pin 1 (White/Orange)

Pin 2 ─────────────────────────► Pin 2 (Orange)

Pin 3 ─────────────────────────► Pin 3 (White/Green)

Pin 4 ─────────────────────────► Pin 4 (Blue)

Pin 5 ─────────────────────────► Pin 5 (White/Blue)

Pin 6 ─────────────────────────► Pin 6 (Green)

Pin 7 ─────────────────────────► Pin 7 (White/Brown)

Pin 8 ─────────────────────────► Pin 8 (Brown)

| Pin | Wire Color |
| --- | --- |
| 1 | White/Orange |
| 2 | Orange |
| 3 | White/Green |
| 4 | Blue |
| 5 | White/Blue |
| 6 | Green |
| 7 | White/Brown |
| 8 | Brown |

Watch a YouTube video demonstrating how to use an RJ45 crimper, then write a step-by-step guide (in your own words) on how to attach an RJ45 connector to a CAT6 cable.

Ans :

Step-by-Step Guide: Attaching an RJ45 Connector to a CAT6 Cable (T568B)

Tools and Materials

- CAT6 Ethernet cable

- RJ45 (CAT6-compatible) connector

- RJ45 crimping tool

- Wire stripper/cutter

- Cable tester (recommended)

Steps

- Cut the cable

  - Cut the CAT6 cable to the required length using the cutter on the crimping tool.

- Strip the outer jacket

  - Remove about 2–3 cm (1 inch) of the outer insulation.

  - Be careful not to damage the inner wires.

- Separate the twisted pairs

  - Untwist the four wire pairs and straighten all eight wires as much as possible.

- Arrange the wires in T568B order

  - Place the wires in this exact sequence:

    - White/Orange

    - Orange

    - White/Green

    - Blue

    - White/Blue

    - Green

    - White/Brown

    - Brown

- Trim the wires

  - Hold the wires firmly together and cut them so they are all the same length (about 12–13 mm exposed).

- Insert the wires into the RJ45 connector

  - Hold the connector with the gold contacts facing up and the clip facing down.

  - Push all eight wires fully into the connector until each wire reaches the end and the cable jacket enters the connector for strain relief.

- Crimp the connector

  - Insert the RJ45 connector into the crimping tool.

  - Squeeze the handles firmly until the crimp is complete.

  - This pushes the metal pins into the wires and secures the cable jacket.

- Repeat for the other end

  - For a straight-through cable, use the same T568B wiring order on the second end.

- Test the cable

  - Connect both ends to a network cable tester.

  - Verify that pins 1 through 8 light up in the correct order, confirming there are no open, crossed, or shorted wires.

Imagine you are setting up a gaming café with 10 PCs for LAN gaming. Decide which cable type (Ethernet or fiber optics) and which tool (RJ45 crimper or fiber cleaver) you would use, and explain your choices in 3-4 sentences.<br><br><em><strong>Hint:</strong> Consider speed, distance, and budget for your answer.</em>

Ans :

For a gaming café with 10 PCs, I would use Ethernet (CAT6) cables because the computers are in the same room, so the cable distance is short. Ethernet provides high speed (1 Gbps or more), low latency, and is much more affordable than fiber optics, making it the best choice for the budget. I would use an RJ45 crimper to attach RJ45 connectors to the CAT6 cables. Fiber optics and a fiber cleaver are better suited for very long-distance connections, such as between buildings or in large data centers, and would be unnecessarily expensive for this setup.