Day – 5 Assi

Connect your laptop and a friend's phone to the same Wi-Fi network, then use the ping command to check if both devices can communicate with each other.<br><br><em><strong>Hint:</strong> On Windows, use 'ping [IP address]' in Command Prompt; on Android, try a ping app or use Termux.</em>

Ans :

Steps

- Connect both devices to the same Wi-Fi

  - Connect your laptop and your friend's phone to the same Wi-Fi network.

- Find the IP address of the phone

  - Android: Go to Settings → Wi-Fi → Tap the connected network → IP Address.

  - Example IP: 192.168.1.105

- Find the IP address of the laptop (optional)

  - Open Command Prompt and type:

ipconfig

  - Note the IPv4 Address (for example, 192.168.1.100).

- Ping the phone from the laptop

  - Open Command Prompt on the laptop.

  - Type:

ping 192.168.1.105

(Replace the IP with your friend's phone's IP.)

- Expected result
If communication is working, you should see something like:

- Pinging 192.168.1.105 with 32 bytes of data:

- Reply from 192.168.1.105: bytes=32 time=5ms TTL=64

- Reply from 192.168.1.105: bytes=32 time=4ms TTL=64

- Ping statistics for 192.168.1.105:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

If the ping fails

You may see:

Request timed out.

or

Destination host unreachable.

Possible reasons:

- The phone blocks ping (many phones do this by default).

- The devices are not connected to the same Wi-Fi.

- A firewall is blocking ICMP (ping) traffic.

- AP/Client Isolation is enabled on the Wi-Fi router, preventing devices from communicating.

Run the traceroute (tracert on Windows or traceroute on Mac/Linux) command to www.youtube.com from your device and note down the number of hops it takes to reach the destination

Ans:

Windows

- Press Win + R, type cmd, and press Enter.

- Run:

tracert www.youtube.com

- Wait until the trace completes.

macOS/Linux

Open Terminal and run:

traceroute www.youtube.com

Example Output

Tracing route to www.youtube.com [142.250.xxx.xxx]

1    <1 ms    <1 ms    <1 ms  192.168.1.1

2     6 ms     5 ms     6 ms  ...

3    10 ms     9 ms    11 ms  ...

...

11    35 ms    34 ms    36 ms  www.youtube.com

Trace complete.

How to find the number of hops

Count the numbered lines. In the example above, the destination is reached in 11 hops.

Disconnect one device from the Wi-Fi, then try pinging it again from your laptop and describe the error message you receive.

Ans :

Steps

- Disconnect the phone (or other device) from the Wi-Fi.

- On your laptop, open Command Prompt.

- Run:

ping <device_IP_address>

Example:

ping 192.168.1.105

Common Error Messages

1. Request timed out. (Most common)

Pinging 192.168.1.105 with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 192.168.1.105:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

Meaning: The device is not responding because it is disconnected, powered off, or unreachable.

2. Destination host unreachable.

Reply from 192.168.1.100: Destination host unreachable.

Meaning: Your laptop or the network cannot find a route to the target device.

Create a simple table listing the IP addresses of all devices connected to your home Wi-Fi (use 'arp -a' on Windows), and identify which device belongs to which person or gadget.

Ans :

Step 1: View connected devices

- Connect your laptop to your home Wi-Fi.

- Open Command Prompt.

- Run:

arp -a

- This will display a list of IP addresses and MAC addresses of devices your computer has recently communicated with.

Step 2: Create the table

Use a table like this and replace the sample data with your own:

| IP Address | MAC Address | Device Name/Owner |
| --- | --- | --- |
| 192.168.1.1 | AA-BB-CC-DD-EE-01 | Wi-Fi Router |
| 192.168.1.2 | AA-BB-CC-DD-EE-02 | My Laptop |
| 192.168.1.5 | AA-BB-CC-DD-EE-03 | My Phone |
| 192.168.1.8 | AA-BB-CC-DD-EE-04 | Father's Phone |
| 192.168.1.10 | AA-BB-CC-DD-EE-05 | Smart TV |

Note

- arp -a only shows devices that your computer has recently communicated with, so it may not list every device on your Wi-Fi.

- If you're unsure which device is which, you can identify them by:

  - Turning a device's Wi-Fi off and on and running arp -a again.

  - Comparing the MAC address with the Wi-Fi or network settings on the device.

If you paste the output of arp -a here, I'll identify each device and make the table for you.

Write a one-page report explaining step-by-step how you set up the network, tested connectivity using ping and traceroute, and what results or issues you observed.<br><br><em><strong>Hint:</strong> Include screenshots or command outputs if possible.</em>

Ans :

Home Network Setup and Connectivity Testing Report

Aim

To set up a home network, verify communication between devices using the ping command, and analyze the network path using the tracert (traceroute) command.

Devices Used

- Windows Laptop

- Android Smartphone

- Home Wi-Fi Router

Procedure

1. Connecting Devices

- Switched on the home Wi-Fi router.

- Connected the Windows laptop to the Wi-Fi network.

- Connected the Android smartphone to the same Wi-Fi network.

- Verified that both devices received IP addresses from the router.

2. Finding the IP Address

- On the laptop, I opened Command Prompt and used:

- ipconfig

- On the smartphone, I checked the Wi-Fi settings to find its IP address.

3. Testing Connectivity Using Ping

I opened Command Prompt on the laptop and ran:

ping 192.168.1.105

(Replace this with your phone's actual IP address.)

Observation:

- The ping command returned replies from the smartphone.

- Packet loss was 0%, indicating successful communication between the devices.

Sample Output:

Reply from 192.168.1.105: bytes=32 time=5ms TTL=64

Reply from 192.168.1.105: bytes=32 time=4ms TTL=64

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

4. Running Traceroute

To check the path to YouTube, I used:

tracert www.youtube.com

Observation:

- The command displayed multiple network hops between my computer and the destination.

- The final hop successfully reached the YouTube server.

5. Testing After Disconnecting a Device

I disconnected the smartphone from the Wi-Fi network and ran the ping command again.

Result:
The command displayed:

Request timed out.

This showed that the phone was no longer reachable because it had disconnected from the network.

Issues Observed

- Some devices may not respond to ping because of firewall or security settings.

- Traceroute may show * * * for some hops if routers block ICMP requests.

- Internet speed and network congestion can affect response time.

Conclusion

The home network was successfully set up, and both devices communicated correctly while connected to the same Wi-Fi network. The ping command confirmed connectivity between devices, while tracert displayed the route taken to reach an internet server. After disconnecting one device, the ping test failed as expected, confirming that network connectivity depends on both devices being connected to the same network.

Screenshots to Attach

- ipconfig output showing the laptop's IP address.

- Phone Wi-Fi settings showing its IP address.

- Successful ping command output.

- tracert www.youtube.com command output.

- Ping output after disconnecting the phone (Request timed out).