Day – 4 Assignment

Given a network address of 192.168.10.0/24, divide it into 4 equal subnets and write down the subnet mask, network address, and broadcast address for each subnet.

Ans :

Given network: 192.168.10.0/24

To create 4 equal subnets, borrow 2 bits:

- New CIDR = /24 + 2 = /26

- Subnet mask = 255.255.255.192

- Block size = 256 − 192 = 64

- Each subnet has 64 total addresses and 62 usable hosts.

Four Equal Subnets

| Subnet | Network Address | Broadcast Address | Usable Host Range |
| --- | --- | --- | --- |
| 1 | 192.168.10.0/26 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 |
| 2 | 192.168.10.64/26 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 |
| 3 | 192.168.10.128/26 | 192.168.10.191 | 192.168.10.129 – 192.168.10.190 |
| 4 | 192.168.10.192/26 | 192.168.10.255 | 192.168.10.193 – 192.168.10.254 |

For one of the subnets you created, assign valid IP addresses to 5 devices (such as a router, two laptops, a printer, and a mobile) and list which IP goes to which device.<br><br><em><strong>Hint:</strong> Make sure not to use the network or broadcast address for any device.</em>

Ans :

Let's use Subnet 1 from the previous question:

- Network: 192.168.10.0/26

- Subnet mask: 255.255.255.192

- Valid host range: 192.168.10.1 – 192.168.10.62

- Broadcast: 192.168.10.63

Device IP Assignment

| Device | Assigned IP Address |
| --- | --- |
| Router | 192.168.10.1 |
| Laptop 1 | 192.168.10.2 |
| Laptop 2 | 192.168.10.3 |
| Printer | 192.168.10.4 |
| Mobile | 192.168.10.5 |

Draw a simple diagram (hand-drawn or digital) showing your subnet plan: label each subnet, the range of IPs, and where each device is connected.

Document step-by-step how you calculated the subnet mask, number of hosts per subnet, and assigned IP addresses for your plan, as if you are explaining it to a friend who is new to subnetting.

Ans :

Step-by-Step Subnetting Calculation

Given Network

- Original Network: 192.168.10.0/24

- Required Subnets: 4

Step 1: Calculate the New Subnet Mask

The original network is /24.

We need 4 subnets.

To find how many bits to borrow:

2² = 4

So, we borrow 2 bits from the host portion.

New prefix:

/24 + 2 = /26

Therefore, the new subnet mask is:

/26 = 255.255.255.192

Step 2: Calculate Hosts Per Subnet

A /26 network leaves:

32 − 26 = 6 host bits

Total IP addresses:

2⁶ = 64 addresses

Two addresses cannot be assigned to devices:

- One is the network address

- One is the broadcast address

Therefore:

Usable hosts = 64 − 2 = 62 hosts

So, each subnet has 62 usable host addresses.

Step 3: Find the Block Size

The last octet of the subnet mask is 192.

Block size:

256 − 192 = 64

Therefore, the subnet addresses increase by 64:

0 → 64 → 128 → 192

Step 4: Create the Four Subnets

Subnet 1

- Network Address: 192.168.10.0

- Usable IPs: 192.168.10.1 – 192.168.10.62

- Broadcast Address: 192.168.10.63

Subnet 2

- Network Address: 192.168.10.64

- Usable IPs: 192.168.10.65 – 192.168.10.126

- Broadcast Address: 192.168.10.127

Subnet 3

- Network Address: 192.168.10.128

- Usable IPs: 192.168.10.129 – 192.168.10.190

- Broadcast Address: 192.168.10.191

Subnet 4

- Network Address: 192.168.10.192

- Usable IPs: 192.168.10.193 – 192.168.10.254

- Broadcast Address: 192.168.10.255

Step 5: Assign IP Addresses to Devices

I selected Subnet 1 for my devices.

Subnet 1 has the usable range 192.168.10.1 – 192.168.10.62.

| Device | IP Address |
| --- | --- |
| Router | 192.168.10.1 |
| Laptop 1 | 192.168.10.2 |
| Laptop 2 | 192.168.10.3 |
| Printer | 192.168.10.4 |
| Mobile | 192.168.10.5 |

All these IP addresses are valid because they are inside the usable host range.

I did not assign:

- 192.168.10.0 → Network address

- 192.168.10.63 → Broadcast address

Final Summary

Original: 192.168.10.0/24
New Mask: /26 = 255.255.255.192
Number of Subnets: 4
Total Addresses per Subnet: 64
Usable Hosts per Subnet: 62
Block Size: 64

Easy Trick to Remember

For /26:

0 → 64 → 128 → 192

And every subnet follows:

Network → Usable Hosts → Broadcast

Example:

0 → 1–62 → 63