Assessment

Section A — Concept Application

You are setting up a new office workstation and need to choose the core hardware for it from a supplier's parts list.

Question: Explain the process of selecting a compatible motherboard, CPU, RAM, and storage device for a new office workstation, and justify how you would confirm they will work together.

Ans :

To select compatible hardware for a new office workstation, I would follow these steps:

- Select the CPU first

  - Decide the required performance for office tasks.

  - Check the CPU socket type (for example, AM5 or LGA 1851).

- Choose a compatible motherboard

  - The motherboard must have the same CPU socket as the processor.

  - Check chipset compatibility and make sure the motherboard supports the CPU generation.

  - Check the required RAM type, such as DDR5.

  - Check that it has the required storage slots, such as M.2 NVMe or SATA.

- Select the RAM

  - Choose the RAM type supported by the motherboard, such as DDR5.

  - Check the maximum supported capacity and supported speed.

  - For an office workstation, for example, 16 GB or 32 GB may be suitable depending on the workload.

- Select the storage device

  - Check whether the motherboard supports M.2 NVMe SSD or SATA drives.

  - For a modern office PC, an NVMe SSD is usually a good choice because of its fast performance.

  - Check the required storage capacity, such as 500 GB or 1 TB.

- Confirm compatibility

  - Check the motherboard manufacturer's CPU Support List.

  - Check the motherboard specifications for supported RAM type, capacity and speed.

  - Check the motherboard manual for supported M.2/SATA storage.

  - Also check the PSU and case compatibility before finalizing the complete workstation.

Example: If the CPU uses an AM5 socket, I would select an AM5 motherboard, DDR5 RAM supported by that motherboard, and an M.2 NVMe SSD supported by its M.2 slot. I would then verify all specifications using the manufacturers' compatibility lists and manuals before purchasing.

You are advising a client whose employees mainly run office and productivity applications on their workstations.

Question: Explain the differences between HDD, SSD, and NVMe storage, and recommend the most suitable option for this workload, giving technical reasons for your choice.

Ans :

| Storage | Technology | Speed | Cost | Main Features |
| --- | --- | --- | --- | --- |
| HDD | Magnetic spinning disks | Slow | Low | Large capacity, mechanical parts |
| SSD (SATA) | Flash memory | Faster than HDD | Medium | No moving parts, reliable and quiet |
| NVMe SSD | Flash memory using PCIe | Fastest | Higher | Very low latency and high read/write speeds |

Recommendation: NVMe SSD

I would recommend an NVMe SSD for office workstations.

Technical reasons:

- Uses the PCIe interface, which provides much higher bandwidth than SATA.

- Has lower latency and faster data access.

- Provides faster Windows booting, application loading, file transfers, and multitasking.

- Has no moving mechanical parts, so it is quieter and generally more resistant to physical shock than an HDD.

- A 500 GB or 1 TB NVMe SSD is generally more than enough for typical office applications and documents.

You are configuring a workstation that will run mildly graphics-intensive design software for several hours a day.

Question: Explain the role of the GPU and the cooling system in maintaining performance and stability under this kind of sustained load.

Ans :

For a workstation running graphics-intensive design software for several hours, both the GPU and cooling system are important for maintaining performance and stability.

1. Role of the GPU

- The GPU (Graphics Processing Unit) processes graphics and visual calculations.

- It helps design software handle 3D models, rendering, animations, textures, and high-resolution displays.

- A suitable dedicated GPU provides better graphics performance and reduces the workload on the CPU.

- The GPU has its own VRAM, which stores graphics data for faster access.

- Under sustained workloads, a capable GPU helps maintain smooth performance.

2. Role of the Cooling System

- Continuous graphics work generates significant heat in the GPU and CPU.

- The cooling system uses fans, heatsinks, and sometimes liquid cooling to remove this heat.

- Good airflow keeps components within their safe operating temperatures.

- If temperatures become too high, the GPU/CPU may throttle (reduce speed), causing performance to drop.

- Proper cooling also helps prevent crashes, unexpected shutdowns, and long-term component damage.

You are assembling a PC from components sourced from three different vendors.

Question: Explain why hardware compatibility matters when assembling a computer, and identify potential issues that could arise from incompatible components.

Ans :

Hardware compatibility is important because all computer components must be able to communicate and work together correctly. When components are purchased from different vendors, their specifications must be checked before assembly.

Why compatibility matters

- CPU and motherboard: The CPU socket and chipset must support the processor.

- RAM and motherboard: The RAM type, such as DDR4 or DDR5, must be supported by the motherboard.

- Storage and motherboard: The motherboard must support the storage interface, such as SATA or M.2 NVMe.

- Power supply (PSU): The PSU must provide enough wattage and have the correct power connectors.

- GPU and motherboard/case: The motherboard must have a suitable PCIe slot, and the GPU must physically fit inside the case.

- Case and motherboard: The motherboard form factor (ATX, Micro-ATX, Mini-ITX) must match the case.

Problems caused by incompatible components

- The computer may not power on or boot.

- The system may fail to detect RAM, CPU, or storage.

- The computer may experience crashes, freezing, or instability.

- Components may overheat because of inadequate cooling or airflow.

- The PSU may be unable to supply enough power.

- A component may not physically fit in the case or have the required connector.

- Some hardware features may not work because of BIOS/firmware compatibility.

You are finalizing the build and need to connect the power supply unit (PSU) to the motherboard, GPU, and storage devices before first boot.

Question: Explain how you would choose an appropriately rated PSU for this build and describe the safety and connection checks you would carry out before powering it on.

Ans ;

When selecting and connecting a Power Supply Unit (PSU), I would make sure it can safely provide enough power to all components.

1. Choosing the correct PSU

- Calculate the total power requirement of the CPU, GPU, motherboard, storage, fans, and other components.

- Choose a PSU with some extra capacity, usually around 20–30% above the estimated maximum load.

- Check that the PSU has the required connectors, such as:

  - 24-pin ATX connector → motherboard

  - 4/8-pin CPU EPS connector → CPU power

  - 6/8-pin PCIe or appropriate GPU power connector → graphics card

  - SATA power connectors → SATA SSD/HDD

- Choose a reliable PSU with an appropriate 80 PLUS efficiency rating and suitable protections.

2. Safety and connection checks

Before powering on:

- Switch off and unplug the PSU from the wall.

- Make sure the PSU voltage/input setting is correct if the model has a manual selector.

- Connect the 24-pin motherboard power connector firmly.

- Connect the CPU EPS power connector near the CPU socket.

- Connect the required GPU power cables securely.

- Connect SATA power to SATA storage devices.

- Ensure no cables are loose or touching fans.

- Check that the motherboard is correctly mounted on standoffs and there are no loose screws causing a short circuit.

- Make sure the CPU cooler and fan are properly installed and connected to the correct fan header.

- Check that RAM, GPU, and storage devices are fully seated.

- Perform a final visual inspection for damaged connectors, exposed wires, or incorrect connections.

You are handing over a freshly assembled workstation to the client and need to confirm it is ready for use.

Question: Explain the checks you would perform to confirm the system passes POST successfully, and what you would do if it did not.

Ans :

POST (Power-On Self-Test) is the process the computer performs when it starts to check basic hardware before loading the operating system.

Checks to confirm POST is successful

- Power on the system and check that fans and indicator lights start normally.

- Check that the system shows the manufacturer/BIOS logo or POST screen.

- Enter the BIOS/UEFI using the appropriate key, such as Delete or F2.

- Confirm that the BIOS detects:

  - CPU

  - RAM and correct memory capacity

  - Storage drive

  - GPU/display output

- Check for any POST error messages or beep codes.

- Confirm that the keyboard and other essential devices are detected.

- Check that CPU temperature and cooling fans appear normal in BIOS.

- Save the settings and allow the system to continue to the operating system.

If POST fails

I would:

- Note any error message, beep code, or diagnostic LED.

- Switch off and disconnect power before working inside the PC.

- Check that the 24-pin motherboard and CPU power connectors are properly connected.

- Reseat the RAM and test one module at a time if necessary.

- Reseat the GPU and check its power connectors.

- Check that the CPU cooler and fan are connected correctly.

- Check that the storage device is properly connected.

- Clear/reset the CMOS/BIOS if appropriate.

- Check CPU and motherboard compatibility and BIOS version.

- Test suspected faulty components individually to isolate the problem.

Section B — Practical Coding Tasks

Task 1: Component Identification & Functions Identify and document the functions of the core hardware components in a desktop system. (Foundational — direct identification of a single concept.)Identify and describe the function of the motherboard, CPU, RAM, GPU, storage device, and cooling system. Note the make/model or key specification of each component where visible. Present the findings as a labeled component list or table.

Ans:

| Component | Function | Make/Model or Key Specification |
| --- | --- | --- |
| Motherboard | The main circuit board that connects and allows communication between the CPU, RAM, GPU, storage, and other components. | Example: ASUS / MSI / Gigabyte motherboard; ATX or Micro-ATX |
| CPU (Processor) | Performs calculations and executes instructions. It is responsible for running applications and processing system tasks. | Example: Intel Core i5 / AMD Ryzen 5 |
| RAM | Provides temporary, high-speed memory for programs and data currently being used by the CPU. | Example: 16 GB DDR4/DDR5 |
| GPU (Graphics Card) | Processes graphics, images, video, and 3D workloads and sends the output to the monitor. | Example: NVIDIA GeForce / AMD Radeon, with dedicated VRAM |
| Storage Device | Permanently stores the operating system, applications, documents, and other files. | Example: 500 GB/1 TB NVMe SSD, SATA SSD, or HDD |
| Cooling System | Removes heat produced by the CPU and other components and helps maintain safe operating temperatures. | Example: CPU heatsink + fan, case fans, or liquid cooler |

Task 2: Motherboard Ports & Socket Identification Identify the motherboard's physical layout, ports, and CPU socket type. (Applied — one concept examined in detail across multiple points.) Identify and label all major motherboard ports (SATA, power, USB headers, etc.). Identify all expansion slots (PCIe, M.2, etc.) and their purpose. Identify the CPU socket type and note which CPU families are compatible with it. Photograph or diagram the motherboard with labels.

Ans :

Task 2: Motherboard Ports & Socket Identification

The motherboard is the main circuit board of a computer. It provides connections for the CPU, RAM, storage devices, expansion cards, USB devices, and power supply.

1. Major motherboard ports and connectors

| Port / Connector | Purpose |
| --- | --- |
| CPU Socket | Holds and connects the CPU to the motherboard. |
| 24-pin ATX Power | Supplies main power from the PSU to the motherboard. |
| 4/8-pin CPU Power (EPS) | Provides dedicated power to the CPU. |
| DIMM Slots | Slots used to install RAM modules. |
| SATA Ports | Connect SATA HDDs and SATA SSDs. |
| M.2 Slot | Used mainly for M.2 NVMe SSDs; some M.2 slots may also support SATA M.2 drives. |
| PCIe x16 Slot | Used mainly for a dedicated GPU and other high-bandwidth expansion cards. |
| PCIe x1/x4 Slots | Used for smaller expansion cards such as Wi-Fi, sound, or network cards. |
| USB Headers | Connect front-panel USB ports to the motherboard. |
| Front-Panel Header | Connects the case's power switch, reset switch, power LED, and HDD LED. |
| CPU_FAN Header | Connects the CPU cooling fan or pump. |
| SYS_FAN Header | Connects cabinet/case fans. |
| Front Audio Header | Connects the case's front headphone and microphone ports. |
| Rear USB Ports | Connect external USB devices such as keyboard, mouse, and USB drives. |
| LAN/Ethernet Port | Provides wired network connectivity. |
| Audio Ports | Connect speakers, headphones, and microphones. |
| HDMI/DisplayPort | Provides display output when supported by the motherboard/CPU. |

2. Expansion slots and their purpose

PCIe slots

- PCIe x16: Mainly used for a graphics card (GPU).

- PCIe x4/x1: Used for expansion cards such as Wi-Fi, sound, capture, or additional network cards.

M.2 slots

- Used for compact storage such as NVMe SSDs.

- NVMe drives communicate through PCIe, providing high storage performance.

3. CPU socket identification

The CPU socket must match the processor.

Examples:

| Socket | Compatible CPU family examples |
| --- | --- |
| AM5 | AMD Ryzen 7000/8000/9000-series desktop CPUs, subject to motherboard BIOS/support |
| LGA1700 | Intel 12th, 13th and 14th Gen Core desktop CPUs, subject to motherboard chipset/BIOS |
| LGA1851 | Intel Core Ultra desktop processors using this socket |

Important: The exact CPU compatibility should always be confirmed using the motherboard manufacturer's CPU support list, because socket compatibility alone does not guarantee that every processor will work.

4. Suggested labeled diagram

For your practical submission, take a clear photograph of the actual motherboard and label it approximately like this:

MOTHERBOARD

┌───────────────────────────────┐

│        CPU SOCKET             │

│          [CPU]                │

│                               │

│  RAM/DIMM SLOTS               │

│  [ ][ ][ ][ ]                │

│                               │

│        M.2 SLOT               │

│        [NVMe]                 │

│                               │

│  PCIe x16  ─────────          │

│  PCIe x1   ───               │

│                               │

│  SATA PORTS  [ ][ ][ ][ ]    │

│                               │

│  24-PIN ATX POWER            │

│                               │

│  CPU_FAN / SYS_FAN            │

│  USB HEADERS                  │

│  FRONT-PANEL HEADER           │

└───────────────────────────────┘

↓

REAR I/O PORTS

USB | LAN | Audio | HDMI/DP

Task 3: RAM & Storage Installation Remove and reinstall a RAM module and a storage device. (Intermediate — two components handled together with correct procedure.) Safely remove the RAM module, noting ESD precautions taken. Reinstall the RAM module correctly and confirm it is seated properly. Remove and reinstall the storage device, noting the connection type used (SATA/NVMe). Confirm both components are detected correctly after reinstallation.

Ans :

Task 3: RAM & Storage Installation

Objective

To safely remove and reinstall a RAM module and a storage device, then verify that both components are detected correctly.

1. Safety and ESD Precautions

Before starting:

- Shut down the computer completely.

- Switch off and unplug the PSU.

- Press the power button for a few seconds to discharge remaining power.

- Wear an anti-static wrist strap or regularly touch a grounded metal part of the case.

- Work on a clean, dry, non-static surface.

- Hold RAM and storage devices by their edges and avoid touching the electrical contacts.

2. RAM Removal

- Locate the RAM/DIMM slots on the motherboard.

- Identify the RAM module to be removed.

- Push the locking clips at the ends of the RAM slot outward.

- Hold the RAM module by its edges.

- Carefully lift it straight out of the slot.

- Place it safely on an anti-static surface.

ESD precaution: I avoided touching the gold contacts and handled the RAM only by its edges.

3. RAM Reinstallation

- Check the notch on the RAM module and align it with the notch in the DIMM slot.

- Insert the RAM evenly into the slot.

- Press firmly on both ends until the locking clips click into place.

- Visually confirm that the module is fully seated and both clips are locked.

Note: The recommended RAM slot depends on the motherboard. For two modules, many motherboards use A2 and B2, but the motherboard manual should be followed.

4. Storage Device Removal and Reinstallation

If using an NVMe SSD:

- Locate the M.2 slot on the motherboard.

- Remove the M.2 retaining screw.

- Carefully lift the SSD and slide it out of the connector.

- Hold it by the edges and place it safely aside.

- Reinsert the SSD into the M.2 slot at an angle.

- Press it down gently and secure it with the retaining screw.

Connection type: M.2 NVMe / PCIe

If using a SATA SSD/HDD:

- Disconnect the SATA data cable from the drive.

- Disconnect the SATA power cable.

- Remove the drive from its mounting position.

- Reinstall the drive.

- Reconnect both SATA data and SATA power cables.

Connection type: SATA

5. Verification After Reinstallation

After completing the installation:

- Reconnect the PSU and power on the computer.

- Enter the BIOS/UEFI using Delete, F2, or the appropriate key.

- Verify that the correct RAM capacity is detected.

- Verify that the storage device/model and capacity are detected.

- Allow the operating system to boot.

- In Windows, check:

  - Task Manager → Performance → Memory for RAM.

  - Disk Management or File Explorer for the storage device.

Practical Record

| Component | Action | Connection Type | Verification |
| --- | --- | --- | --- |
| RAM | Removed and reinstalled | DIMM slot | Detected in BIOS/Windows |
| Storage | Removed and reinstalled | M.2 NVMe / SATA | Detected in BIOS/Windows |

Result

RAM and storage were safely removed and reinstalled using proper ESD precautions. Both components were seated correctly and verified as detected by the system.

Task 4: Full Desktop PC Assembly Assemble a desktop PC from the provided components and document the process. (Integrative — combines all Section B skills into one build.) Assemble the PC using the provided motherboard, CPU, RAM, storage, GPU, and PSU. Follow correct installation sequence and ESD/safety precautions throughout. Document each step with labeled photos or diagrams. Power on the system and confirm it passes POST successfully

Ans :

Here is a simple Task 4 practical write-up you can use in your assignment/report.

Task 4: Full Desktop PC Assembly

Objective:
To assemble a complete desktop computer using the provided motherboard, CPU, RAM, storage device, GPU, and PSU, following proper safety and ESD precautions.

Components Used

- Motherboard

- CPU/Processor

- CPU Cooler

- RAM module

- Storage device (SATA SSD/HDD or NVMe SSD)

- Graphics Card (GPU)

- Power Supply Unit (PSU)

- PC Cabinet

- SATA cable, if required

- Power cables

Assembly Procedure

Step 1: Prepare the workspace

- Switch off and unplug all equipment.

- Keep the work area clean and dry.

- Use an ESD wrist strap or regularly touch a grounded metal part of the cabinet.

- Handle components by their edges and avoid touching gold contacts.

Photo 1: Clean workspace and components.

Step 2: Install the CPU

- Place the motherboard on an anti-static surface.

- Open the CPU socket retention mechanism.

- Align the CPU's marking/notch with the socket marking.

- Carefully place the CPU into the socket.

- Lock the retention mechanism.

Photo 2: CPU correctly installed in the motherboard.

Step 3: Install the CPU cooler

- Apply thermal paste if it is not already pre-applied.

- Position the CPU cooler over the CPU.

- Secure the cooler properly.

- Connect the cooler's fan cable to the CPU_FAN header.

Photo 3: CPU cooler installed.

Step 4: Install RAM

- Open the RAM slot clips.

- Align the notch on the RAM module with the slot.

- Press the RAM down evenly until the clips lock.

- Confirm that the module is fully seated.

Photo 4: RAM installed correctly.

Step 5: Install storage

- For NVMe SSD: Insert it into the M.2 slot at an angle and secure it with the screw.

- For SATA SSD/HDD: Mount the drive in the appropriate bay and connect the SATA data cable to the motherboard and SATA power cable from the PSU.

Photo 5: Storage device installed.

Step 6: Install the motherboard into the cabinet

- Install the I/O shield if the motherboard requires a separate one.

- Place the motherboard onto the correct standoffs.

- Align the motherboard with the I/O opening.

- Secure it using the appropriate screws.

Photo 6: Motherboard secured inside the cabinet.

Step 7: Install the GPU

- Remove the required expansion-slot covers.

- Insert the GPU into the appropriate PCIe x16 slot.

- Press it firmly until the slot latch engages.

- Secure the GPU to the cabinet with a screw.

- Connect the required PCIe power cable from the PSU.

Photo 7: GPU installed.

Step 8: Install and connect the PSU

- Place the PSU in its designated position.

- Secure it with screws.

- Connect:

  - 24-pin ATX → motherboard

  - 4/8-pin CPU EPS → CPU power connector

  - PCIe power → GPU, if required

  - SATA power → SATA storage, if required

Photo 8: PSU and power cables connected.

Step 9: Connect front-panel and other cables
Connect the cabinet's:

- Power switch

- Reset switch, if present

- Power LED

- HDD LED

- Front USB

- Front audio

- Case fans

Ensure cables are properly connected and do not interfere with the fans.

Photo 9: Front-panel and other cables connected.

Step 10: Final inspection
Before powering on:

- Check that RAM is properly seated.

- Check that GPU is properly seated.

- Check CPU cooler connection.

- Check the 24-pin and CPU power connectors.

- Check storage connections.

- Make sure no loose screws or cables are inside the cabinet.

- Ensure all fans can rotate freely.

Photo 10: Final assembled PC.

Step 11: Power On and POST Test

- Connect the monitor, keyboard, and power cable.

- Turn on the PSU.

- Press the PC power button.

- Check that the CPU, case, and GPU fans start spinning.

- Check the monitor for the manufacturer's logo or BIOS/UEFI screen.

- Enter BIOS/UEFI if required and verify that the CPU, RAM, and storage device are detected.

POST Result:
The system powered on successfully and completed POST (Power-On Self-Test) without any error. The installed hardware was detected correctly in the BIOS/UEFI.

Result

The desktop PC was successfully assembled using the motherboard, CPU, RAM, storage device, GPU, and PSU. Proper ESD and safety precautions were followed during the assembly. After powering on the system, it successfully passed POST and the installed components were detected correctly.

Section C — Mini Capstone Project

Mini Project: Workstation Deployment & Boot-Failure Diagnosis Objective: ABC Solutions is setting up a new computer lab. After assembling several systems, some computers fail to start while others display hardware detection errors, and the IT team suspects incorrect installation or compatibility issues. Build (or inspect) a workstation, diagnose the likely causes of these failures, and produce a diagnostic and installation-verification report — combining component selection, installation procedure, and compatibility troubleshooting from Section B.

Your project must: Identify the three hardware components you would check first when a newly assembled PC fails to boot, and explain why. Explain how motherboard–CPU compatibility affects system functionality, referencing the specific system you assembled or inspected. Describe how you would verify that RAM and storage devices are installed correctly, including what BIOS/POST should confirm. Produce a hardware-installation checklist of at least three steps to prevent similar failures in future deployments. Provide a technical justification for using SSDs instead of HDDs in modern office workstations, tying it back to the lab's use case.

Ans :

Mini Project: Workstation Deployment & Boot-Failure Diagnosis

1. Introduction

ABC Solutions is setting up a new computer lab with multiple desktop workstations. During installation, some computers failed to start, while others showed hardware detection errors.

The main possible reasons are:

- Incorrect component installation

- Loose connections

- Incompatible hardware

- Incorrect BIOS/UEFI settings

- Faulty RAM or storage devices

This project focuses on assembling/inspecting a workstation, checking hardware compatibility, verifying installation, and diagnosing boot failures.

2. Three Hardware Components to Check First When a PC Fails to Boot

When a newly assembled PC does not boot, I would check these three components first:

1. Power Supply Unit (PSU)

Why check it first?

- The PC cannot start without proper power.

- A loose 24-pin motherboard connector or 4/8-pin CPU power connector can prevent booting.

- A faulty or insufficient PSU may cause the system to shut down or show no response.

Check:

- PSU power switch is ON.

- AC power cable is connected.

- 24-pin ATX connector is properly connected.

- 4/8-pin CPU power connector is properly connected.

2. RAM

Why check RAM?

- Incorrectly installed or incompatible RAM can prevent POST.

- A loose RAM module may cause a black screen or memory error/beep code.

Check:

- Remove and reinstall the RAM.

- Make sure the RAM notch matches the slot.

- Ensure both side clips lock properly.

- Try one RAM module at a time if multiple modules are installed.

3. CPU and CPU Installation

Why check the CPU?

- The CPU is required for the system to perform POST.

- An incompatible CPU or incorrectly installed CPU can cause the system not to boot.

- The CPU power connector and CPU cooler connection must also be checked.

Check:

- CPU is compatible with the motherboard socket.

- CPU is installed in the correct orientation.

- CPU power connector is connected.

- CPU cooler is properly installed.

- CPU_FAN cable is connected.

3. Motherboard–CPU Compatibility

Motherboard and CPU compatibility is very important because the CPU must be supported by the motherboard's socket, chipset, BIOS/UEFI, and CPU generation.

For example, suppose the workstation uses:

| Component | Example |
| --- | --- |
| Motherboard | ASUS B550 motherboard |
| CPU | AMD Ryzen 5 5600 |
| RAM | 16 GB DDR4 |
| Storage | 500 GB NVMe SSD |
| GPU | NVIDIA/AMD PCIe GPU |
| PSU | 550 W PSU |

The AMD Ryzen 5 5600 requires a compatible AM4 socket motherboard and appropriate BIOS support. If an incompatible CPU is installed, the system may receive power but fail to complete POST.

Therefore, before assembly, I would verify:

- CPU socket compatibility

- Motherboard chipset support

- CPU generation support

- Required BIOS version

- RAM type supported by the motherboard

- PSU power requirements

Conclusion: Even if the CPU physically fits the socket, it does not necessarily mean that the motherboard can operate with it.

4. RAM Installation and Verification

Correct RAM Installation

- Switch off the PC and disconnect the power.

- Open the RAM slot clips.

- Align the notch on the RAM module with the notch in the motherboard slot.

- Press the RAM down evenly until the clips lock.

- Check that the RAM is completely seated.

BIOS/POST Verification

After powering on:

- The system should successfully complete POST.

- BIOS/UEFI should display the installed RAM capacity.

- For example, if 16 GB RAM is installed, BIOS should approximately show 16 GB of system memory.

- If the RAM is not detected, check seating, slot compatibility, or RAM compatibility.

Example:

Installed RAM: 16 GB DDR4
BIOS detected RAM: 16 GB DDR4
Status: PASS

5. Storage Installation and Verification

The workstation may use an NVMe SSD or SATA SSD.

NVMe SSD

- Locate the motherboard's M.2 slot.

- Insert the NVMe SSD at a slight angle.

- Push it down carefully.

- Secure it with the M.2 screw.

- Check that it is firmly installed.

SATA SSD

- Mount the SSD securely.

- Connect the SATA data cable to the motherboard.

- Connect SATA power from the PSU.

- Check both connections.

BIOS/UEFI Verification

After powering on:

- Enter BIOS/UEFI.

- Open the storage/NVMe/SATA information section.

- Verify that the SSD model and capacity are detected.

Example:

Storage installed: 500 GB NVMe SSD
BIOS detected: 500 GB NVMe SSD
Status: PASS

If the storage device is not detected:

- Check the physical connection.

- Try another SATA port if using SATA.

- Reseat the NVMe SSD.

- Check BIOS storage settings.

- Verify motherboard compatibility.

6. Diagnostic Process for Boot Failure

The following troubleshooting sequence can be used when a workstation fails to boot:

Power ON → Check PSU → Check motherboard power → Check CPU → Check RAM → Check GPU → Check storage → Check BIOS/UEFI → Check POST error codes

Common Symptoms and Causes

| Symptom | Possible Cause | Action |
| --- | --- | --- |
| No power | PSU/cable problem | Check PSU and power connections |
| Fans spin but no display | RAM/GPU/CPU | Reseat components |
| Memory error | RAM incorrectly installed | Reinstall/test RAM |
| CPU error | CPU incompatibility | Check socket/BIOS support |
| Storage not detected | Loose/incorrect connection | Reconnect/reseat storage |
| Repeated restart | PSU/RAM/CPU issue | Test components |
| BIOS cannot detect SSD | SSD/slot/connection issue | Check M.2/SATA connection |

7. Hardware-Installation Checklist

The following checklist can prevent similar failures during future deployments:

Pre-Installation

☐ Check motherboard and CPU compatibility.
☐ Check RAM type and supported capacity.
☐ Check storage interface: SATA or NVMe.
☐ Check PSU wattage and required connectors.
☐ Use ESD protection.

Installation

☐ Install CPU in the correct orientation.
☐ Install CPU cooler and connect CPU_FAN.
☐ Seat RAM correctly until the clips lock.
☐ Install and secure the storage device.
☐ Install GPU correctly in the PCIe slot.
☐ Connect 24-pin ATX and CPU power connectors.
☐ Connect SATA power/data cables when required.

Post-Installation

☐ Perform a visual inspection for loose cables.
☐ Power on and confirm POST.
☐ Enter BIOS/UEFI and verify CPU and RAM.
☐ Verify that the storage device is detected.
☐ Install the operating system and required drivers.

8. Why SSDs Are Better Than HDDs for Modern Office Workstations

For ABC Solutions' computer lab, SSDs are technically more suitable than traditional HDDs.

Comparison

| Feature | SSD | HDD |
| --- | --- | --- |
| Speed | Very fast | Slower |
| Moving parts | No | Yes |
| Boot time | Faster | Slower |
| Application loading | Faster | Slower |
| Noise | Silent | Mechanical noise |
| Shock resistance | Better | Lower |
| Power consumption | Generally lower | Generally higher |
| Reliability for mobile handling | Better | More sensitive to physical shock |

Lab Use Case

Office workstations may be used for:

- Web browsing

- Microsoft Office

- Programming

- Online meetings

- Database applications

- File management

- Cybersecurity/networking software

These activities benefit from faster boot times and faster application/file access.

For example, if many students are starting computers simultaneously in the lab, SSD-based systems can provide faster startup and application loading, improving overall productivity and reducing waiting time.

Therefore, I would select NVMe SSDs where supported because they provide high-speed storage without the mechanical moving parts found in HDDs.

9. Final Conclusion

The workstation deployment process requires careful component selection, compatibility checking, correct installation, and POST/BIOS verification.

The first components I would check during a boot failure are the PSU, RAM, and CPU, because problems with these components can prevent the system from completing POST.

Motherboard–CPU compatibility must be confirmed before assembly, including the socket, chipset, CPU generation, and BIOS support. RAM and storage should then be physically checked and verified through BIOS/UEFI.

By following a proper installation checklist and using SSDs instead of HDDs, ABC Solutions can build faster, more reliable, and easier-to-maintain workstations for its computer lab.

Section D — AI-Augmented Learning

STEP 1 · BUILD WITH AI Use an AI tool of your choice (ChatGPT, Claude, Copilot, etc.) to help you produce a step-by-step troubleshooting checklist that: Guides a junior technician through diagnosing a desktop PC that fails to boot immediately after assembly, in the correct priority order. Covers at minimum the motherboard, RAM, storage, and PSU as possible failure points. Is presented as a clear checklist or simple decision flow a technician could follow on-site. Is applied to the ABC Solutions scenario from Section C to show which step would have caught the reported issue.

STEP 2 · TEST & DEBUG (WITHOUT AI) Then, working without AI, review the checklist and find at least one step that is missing, incorrectly ordered, or unsafe (for example, skipping ESD precautions or checking software before physical connections). Fix it yourself.

Ans :

STEP 1 · BUILD WITH AI

AI-Assisted Desktop PC Boot Troubleshooting Checklist

Scenario: ABC Solutions has assembled new desktop computers. Some systems fail to boot immediately after assembly. The checklist below guides a junior technician through the diagnosis in the correct priority order.

Safety First

☐ 1. Disconnect AC power before opening the PC.
☐ Wear/use ESD protection such as an anti-static wrist strap.
☐ Work on a clean, dry, non-static surface.
☐ Do not touch component contacts or pins unnecessarily.

Boot-Failure Decision Checklist

Step 1: Check External Power

☐ Is the power cable connected correctly?
☐ Is the wall socket/UPS switched ON?
☐ Is the PSU switch set to ON (I)?

If NO → Correct the power connection and test again.
If YES → Continue to Step 2.

Step 2: Check PSU Connections

☐ Check the 24-pin ATX motherboard power connector.
☐ Check the 4/8-pin CPU power connector.
☐ Check GPU power cables if the GPU requires them.
☐ Check that the PSU has sufficient wattage for the components.

If a connection is loose → Reconnect it and test.
If connections are correct → Continue to Step 3.

Step 3: Check Motherboard

☐ Check that the motherboard is correctly mounted on the cabinet standoffs.
☐ Check for loose screws or possible short circuits.
☐ Check motherboard diagnostic LEDs/beep codes if available.
☐ Check that the CPU_FAN connector is connected.
☐ Confirm that the motherboard supports the installed CPU and RAM.

If motherboard installation is incorrect → Correct it and test again.
If correct → Continue to Step 4.

Step 4: Check RAM

☐ Switch off and disconnect power.
☐ Remove and reseat the RAM module.
☐ Confirm that the RAM notch matches the slot.
☐ Ensure the retaining clips lock into position.
☐ If there are multiple RAM modules, test one module at a time in the recommended slot.

If POST succeeds → RAM installation was likely the problem.
If POST still fails → Continue to Step 5.

Step 5: Check CPU

☐ Verify that the CPU is installed in the correct orientation.
☐ Check CPU socket/pins for damage.
☐ Confirm CPU power is connected.
☐ Confirm the CPU cooler is installed correctly.
☐ Verify motherboard–CPU compatibility and BIOS support.

If CPU/compatibility is incorrect → Correct or replace the incompatible component.
If correct → Continue to Step 6.

Step 6: Check GPU/Display

☐ Make sure the GPU is properly seated in the PCIe slot.
☐ Connect required GPU power cables.
☐ Check the monitor cable.
☐ If supported, test using integrated graphics.

If display appears → The GPU/display connection was the likely problem.
If no display → Continue to Step 7.

Step 7: Check Storage

☐ Check whether the SSD/HDD is securely installed.
☐ For NVMe, reseat the SSD in the M.2 slot.
☐ For SATA storage, check both SATA data and SATA power cables.
☐ Enter BIOS/UEFI and check whether the storage device is detected.

Important: A missing storage device usually does not prevent the PC from reaching BIOS/UEFI. It may instead cause a "No boot device" or similar error after POST.

If storage is not detected → Reseat/check connections and BIOS settings.
If storage is detected → Continue to Step 8.

Step 8: Check BIOS/UEFI and Boot Device

☐ Enter BIOS/UEFI.
☐ Confirm CPU and RAM are detected.
☐ Confirm the storage device is detected.
☐ Check the boot order.
☐ Confirm the correct operating-system drive is selected.

If BIOS works but Windows/Linux does not start → investigate the boot drive/OS rather than treating it as a basic power failure.

Simple Decision Flow

PC does not boot

↓

1. Power available?
→ NO → Check wall power, cable and PSU switch
→ YES ↓

2. PSU connectors connected?
→ NO → Connect 24-pin + CPU 4/8-pin
→ YES ↓

3. Motherboard installed correctly?
→ NO → Correct mounting/short circuit
→ YES ↓

4. RAM detected/POST successful?
→ NO → Reseat/test RAM
→ YES ↓

5. CPU compatible and correctly installed?
→ NO → Correct CPU installation/compatibility
→ YES ↓

6. Display/GPU working?
→ NO → Check GPU, power and display cable
→ YES ↓

7. Storage detected in BIOS?
→ NO → Check NVMe/SATA installation
→ YES ↓

8. Correct boot device selected?
→ NO → Correct boot order
→ YES → Investigate OS/software boot problem

Applying the Checklist to ABC Solutions

In the ABC Solutions scenario, the reported problem was hardware detection errors after assembly.

The checklist would first verify power and motherboard connections, then RAM and CPU, before checking storage.

Example Diagnosis

Suppose one ABC Solutions workstation showed:

PC powers ON → POST occurs → BIOS does not detect the NVMe SSD

Following the checklist:

Step 1: Power → PASS
Step 2: PSU → PASS
Step 3: Motherboard → PASS
Step 4: RAM → PASS
Step 5: CPU → PASS
Step 6: Display → PASS
Step 7: Storage → FAIL

The technician would then reseat the NVMe SSD and verify the M.2 connection.

Therefore, Step 7 would have caught the reported storage-detection issue.

STEP 2 · TEST & DEBUG — WITHOUT AI

After reviewing the AI-generated checklist manually, I identified one important issue:

Problem Found

The original troubleshooting process could be improved because GPU/display checking was placed before storage, even though the exact symptom should determine the next step. Also, the safety instructions should be treated as a mandatory condition before reseating internal components, not merely as an introductory note.

My Correction

I changed the checklist so that:

- Safety/ESD precautions are mandatory before opening the case.

- Power and PSU connections are checked before component reseating.

- Motherboard/CPU/RAM/POST are checked before storage.

- Storage is checked after confirming that the system can reach BIOS/UEFI.

- OS/software troubleshooting is performed only after basic hardware and BIOS checks pass.