Day – 3 Assi

List 5 common BIOS or POST error messages you might see when starting a PC and write what hardware or configuration issue each one usually points to.

| BIOS/POST Error Message | Usually Indicates |
| --- | --- |
| 1. CMOS Checksum Error | CMOS battery is weak/dead or BIOS settings are corrupted/reset. |
| 2. Keyboard Error / Keyboard Not Detected | Keyboard is disconnected, faulty, or the USB/PS/2 port has a problem. |
| 3. No Boot Device Found | HDD/SSD is not detected, boot drive is faulty, or the boot order is incorrect. |
| 4. Memory Error / RAM Error | RAM is loose, incompatible, faulty, or the RAM slot has a problem. |
| 5. CPU Fan Error | CPU fan is disconnected, not spinning, blocked by dust, or the BIOS fan setting is incorrect. |

Given this scenario: On boot, a PC beeps 3 times in a repeating pattern and does not display anything on the screen. Research and write what this POST beep code typically means for an AMI BIOS, and suggest two troubleshooting steps.

For an AMI BIOS, 3 repeating beeps typically indicate a memory/RAM problem (often a failure in the first 64 KB of system memory, depending on the AMI BIOS version).

Troubleshooting Steps

- Reseat the RAM

  - Turn off and unplug the PC.

  - Remove the RAM module and carefully reinstall it firmly in the slot.

  - Start the PC again and check whether the beeps stop.

- Test the RAM

  - If there are multiple RAM sticks, remove them and test one stick at a time in the recommended slot.

  - If one stick works and another doesn't, the faulty RAM module may need replacement.

Access the BIOS setup utility on any available PC or virtual machine, take a screenshot of the main BIOS screen, and identify the motherboard manufacturer and BIOS version.

Steps

- Restart the PC.

- Immediately press Delete, F2, or the key shown on the startup screen to enter BIOS/UEFI.

- On the Main / Information / System Information page, look for:

  - Motherboard/Manufacturer

  - BIOS Version

- Take a screenshot using your phone or the VM's screenshot function.

Example practical record

| Item | Information |
| --- | --- |
| Motherboard Manufacturer | ASUS |
| BIOS Version | American Megatrends Inc. 3205 |
| BIOS Mode | UEFI |

You are given a screenshot of a POST error message that says 'CMOS checksum error – Defaults loaded.' Explain what this means and describe how you would resolve this issue in a real system.<br><br><em><strong>Hint:</strong> Think about the CMOS battery and BIOS settings reset.</em>

CMOS Checksum Error – Defaults Loaded

Meaning:
This message means the BIOS detected that the CMOS settings/data are incorrect or corrupted. Because of this, the BIOS has loaded its default settings instead of the previously saved settings.

Common Cause

The most common hardware cause is a weak or dead CMOS battery (usually a CR2032). It can also happen after a BIOS reset or if BIOS settings become corrupted.

How to Resolve It

- Enter BIOS/UEFI

  - Press Delete, F2, or the key shown during startup.

- Check the BIOS settings

  - Set the correct date and time.

  - Check the boot order and other required settings.

- Save and restart

  - Choose Save & Exit and reboot.

  - If the error does not return, the issue may have been temporary.

- Replace the CMOS battery

  - If the error returns repeatedly, shut down and unplug the PC.

  - Replace the motherboard's CR2032 CMOS battery with a suitable new one.

  - Enter BIOS again and restore the required settings.

- If the error continues

  - Check for BIOS/firmware issues or a motherboard problem.

  - Consult the motherboard manufacturer's service documentation.