Day – 4 Assi

Open your desktop or laptop, locate the RAM slots, and remove and re-insert the RAM module to simulate reseating — then document each step you took and any issues you observed during the process.

RAM Reseating Practical

Objective:
To remove and re-insert a RAM module to ensure it is properly connected.

Steps:

- Shut down the computer

  - Turn off Windows completely.

  - Disconnect the power cable/charger.

- Discharge residual power

  - Press and hold the power button for about 10 seconds after unplugging the system.

- Open the computer case

  - Remove the side panel of the desktop.

  - For a laptop, remove the back cover if the RAM is accessible.

- Locate the RAM slots

  - Find the long memory slots on the motherboard.

  - Identify the RAM module.

- Remove the RAM

  - Open the locking clips on both ends of the RAM slot.

  - Hold the RAM by its edges and gently remove it.

  - Avoid touching the gold contacts.

- Inspect the RAM and slot

  - Check for dust, physical damage, or damaged contacts.

  - Observation: No physical damage found. (Change this if your actual observation is different.)

- Re-insert the RAM

  - Align the notch on the RAM with the notch in the slot.

  - Press the module down evenly until the locking clips click into place.

- Close the case and reconnect power

  - Reconnect the power cable/charger.

- Start the computer

  - Check whether the PC boots normally.

  - Check for display and listen for unusual POST beep codes.

Observation Table

| Check | Observation |
| --- | --- |
| RAM module | No visible damage |
| RAM slot | No visible dust/damage |
| RAM seating | Properly locked into slot |
| Display after reseating | Normal display |
| POST beeps | No unusual beeps |
| System boot | Successfully booted |

Create a troubleshooting checklist for diagnosing a faulty GPU in a gaming PC, including at least 5 steps from initial symptom identification to confirming the fix.<br><br><em><strong>Hint:</strong> Think about display issues, driver checks, and physical connections.</em>

| Step | Check | What to Look For | Result |
| --- | --- | --- | --- |
| ☐ 1. Identify the symptom | Start the PC and observe the display | No display, black screen, artifacts, flickering, crashes, or low graphics performance | __________ |
| ☐ 2. Check the monitor & cable | Inspect HDMI/DisplayPort cable and monitor input | Loose/damaged cable or incorrect monitor input | __________ |
| ☐ 3. Check GPU physical connection | Power off PC and inspect the GPU | GPU properly seated in PCIe slot; no visible damage | __________ |
| ☐ 4. Check GPU power cables | Inspect PCIe power connectors | 6-pin/8-pin connectors securely connected; no loose cables | __________ |
| ☐ 5. Check cooling | Inspect GPU fans and heatsink | Fans spinning when required, no excessive dust, and no unusual overheating | __________ |
| ☐ 6. Check the display driver | In Windows, check Device Manager/display settings | GPU detected correctly; no warning/error symbol; driver is functioning | __________ |
| ☐ 7. Test with another connection | Try another display cable/port or monitor | Determines whether the problem is the GPU or display equipment | __________ |
| ☐ 8. Test the GPU | Run a normal game or graphics test | No artifacts, crashes, black screens, or abnormal behavior | __________ |
| ☐ 9. Confirm the fix | Restart the PC and test again | System boots normally and gaming works without GPU-related problems | __________ |

Simulate a storage issue by unplugging and replugging a SATA or NVMe drive (SSD/HDD) on a test system, then describe the error messages or BIOS indicators you see before and after reconnecting.

Storage Issue Simulation — SATA SSD/HDD

Important: Do this only on a test system or a drive that does not contain important data. Shut the PC down completely and disconnect power before unplugging a SATA drive.

1. Before disconnecting the drive

- Start the PC normally.

- Enter BIOS/UEFI using Delete, F2, etc.

- Open Storage / SATA Information / Boot.

- Confirm that the SSD/HDD is detected.

Example:

- SATA SSD — 500 GB — Detected

- Boot option: Windows Boot Manager

2. Disconnect the SATA drive

- Shut down the PC.

- Unplug the SATA data cable or SATA power cable from the test drive.

- Start the PC.

3. Observe the error

The BIOS may show something similar to:

SATA SSD — Not Detected

If it was the boot drive, the PC may display:

- No Boot Device Found

- Operating System Not Found

- Reboot and Select Proper Boot Device

The exact message depends on the motherboard and BIOS.

4. Reconnect the drive

- Shut down the PC again.

- Reconnect the SATA data and power cables firmly.

- Start the PC and enter BIOS/UEFI.

After reconnecting:

- SATA SSD — 500 GB — Detected

- Windows Boot Manager — Available

The operating system should then boot normally.

Observation Table

| Stage | BIOS/Screen Observation |
| --- | --- |
| Before unplugging | SSD/HDD detected |
| After unplugging | Drive not detected |
| Boot attempt | “No Boot Device Found” / similar message |
| After reconnecting | SSD/HDD detected again |
| Final result | Operating system boots normally |

Given a scenario where a PC randomly restarts, list which component (RAM, GPU, storage) you would check first and explain your reasoning, then outline the specific diagnostic steps you would perform for that component.

Random PC Restarts — Which Component to Check First?

I would check the RAM first.

Reason: Faulty or poorly seated RAM can cause sudden system instability, freezes, BSODs, and random restarts. It is also relatively easy to inspect and reseat compared with diagnosing a GPU or storage device.

Diagnostic Steps for RAM

- Power off the PC

  - Shut down the computer and disconnect the power cable.

- Inspect the RAM

  - Open the case and check that the RAM is properly seated.

  - Look for dust, physical damage, or a damaged slot.

- Reseat the RAM

  - Remove the RAM module carefully.

  - Reinsert it firmly until the locking clips click into place.

- Test one RAM module at a time

  - If there are multiple RAM sticks, remove all but one.

  - Boot the PC and observe whether the random restart occurs.

  - Repeat with each RAM stick.

- Try another RAM slot

  - If a module works in one slot but not another, the slot or motherboard may be the problem.

- Check the result

  - Run the PC normally or perform a typical workload.

  - If restarts stop with one particular module removed, that RAM module may be faulty.