Day – 2 Assi

List 5 common hardware issues you might encounter in a desktop PC and write the typical symptom for each (for example: 'No display on monitor' — symptom: blank screen, no POST beep).

| Hardware Issue | Typical Symptom |
| --- | --- |
| 1. No display on monitor | Blank screen, monitor shows "No Signal," and the computer may produce no POST (Power-On Self-Test) beep. |
| 2. Faulty RAM (Memory) | Computer fails to boot, continuous or repeated beep codes, random crashes, or frequent Blue Screen of Death (BSOD). |
| 3. Hard drive/SSD failure | Operating system does not load, "Boot device not found" error, slow performance, clicking noises (HDD), or missing files. |
| 4. Power Supply Unit (PSU) failure | PC does not power on at all, no lights or fans, or the system suddenly shuts down or restarts unexpectedly. |
| 5. Overheating CPU or cooling fan failure | Computer becomes very hot, fan runs loudly or stops spinning, frequent freezing, automatic shutdowns, or poor performance. |

Given this scenario: A laptop is not charging even when plugged in. Write down a step-by-step troubleshooting plan using only basic tools (no software diagnostics), and explain what you would check at each step.

Laptop Not Charging — Step-by-Step Hardware Troubleshooting

Use only basic tools such as the charger, another compatible charger if available, a multimeter, and a screwdriver if needed.

- Check the power outlet

  - Plug another device into the same socket.

  - Check: If the other device also doesn't work, the problem may be the wall socket.

- Inspect the charger and cable

  - Look for cuts, burns, bent pins, loose connections, or damaged insulation.

  - Check: Make sure the adapter and cable are firmly connected.

- Check the charger LED

  - If the adapter has a power LED, see whether it is ON.

  - Check: No LED may indicate a faulty adapter or power connection.

- Check the laptop charging port

  - Look inside the DC charging port for dust, damage, or bent/loose contacts.

  - Gently move the plug.

  - Check: If charging starts/stops when the plug moves, the port may be loose or damaged.

- Try a known-good compatible charger

  - Use another charger with the same voltage, connector, and suitable current rating.

  - Check: If the laptop charges with the other charger, the original charger is likely faulty.

- Remove the battery if it is removable

  - Turn off the laptop, remove the battery, connect the charger, and try powering it on.

  - Check: If it works without the battery, the battery may be faulty.

- Check the battery physically

  - Look for swelling, leakage, unusual heat, or physical damage.

  - Check: A swollen or damaged battery should not be used and should be replaced safely.

- Test the charger with a multimeter

  - Set the multimeter to the appropriate DC voltage range.

  - Measure the charger's output according to its label and connector design.

  - Check: A significantly incorrect or zero output indicates a possible faulty charger.

- Final hardware check

  - If the charger is good but the laptop still doesn't charge, the likely causes include the charging jack, battery, charging circuit, or motherboard.

  - At this point, professional hardware repair may be required.

Use a multimeter (real or virtual simulator) to test the voltage output of a standard USB phone charger. Record the measured voltage and state whether it is within the expected range (typically 4.75V to 5.25V for USB).

USB Charger Voltage Test

Tool: Digital multimeter
Mode: DC Voltage (V⎓)

Steps:

- Plug the USB phone charger into a wall socket.

- Set the multimeter to DC voltage.

- Put the black probe on the USB ground (GND).

- Put the red probe on the USB +5V (VBUS) pin.

- Read the voltage on the multimeter.

Example observation:

| Test | Measured Voltage | Expected Range | Result |
| --- | --- | --- | --- |
| USB charger output | 5.08 V DC | 4.75–5.25 V | PASS — Normal |

A PC is randomly shutting down. Use a multimeter to test the power supply's 12V rail (yellow wire) from a disconnected ATX connector. Describe the steps and write down the voltage you find. If the voltage is below 11.5V, what hardware issue could this indicate?<br><br><em><strong>Hint:</strong> Remember to set the multimeter to DC voltage and probe between the yellow and black wires.</em>

Steps:

- Shut down the PC and unplug the PSU from the wall.

- Disconnect the ATX power connector from the motherboard.

- Set the multimeter to DC Voltage (V⎓).

- Identify the yellow wire (+12V) and black wire (GND).

- Reconnect the PSU to AC power without reconnecting the ATX connector to the motherboard.

- Turn the PSU on if it has a power switch.

- Place the black probe on the black wire and the red probe on the yellow wire.

- Read and record the voltage.

Example measurement:

| Test | Measured Voltage | Expected | Result |
| --- | --- | --- | --- |
| PSU +12V rail | 11.2 V DC | Around 12 V | Below normal |