Day – 1 Assi

List three real-life scenarios where a multimeter would be used to troubleshoot a computer or smartphone issue, and explain in one line how the multimeter helps in each case.

Ans :

- Computer won't power on – Use the multimeter to measure the power supply's output voltages and verify it is delivering the correct power to the motherboard.

- Smartphone not charging – Use the multimeter to check the charger, USB cable, or battery voltage to identify whether the charging components are functioning properly.

- Laptop or smartphone has no response after a drop or repair – Use the multimeter's continuity mode to detect broken traces, blown fuses, or damaged connections on the circuit board.

Watch a short YouTube video demo of a POST card in action (search for 'POST card motherboard test'), then write a brief summary (3-4 lines) describing what the POST card displays and what problem it helped identify in the video.<br><br><em><strong>Hint:</strong> Focus on the codes shown and how they relate to hardware issues.</em>

Ans:

The POST card displayed hexadecimal POST (Power-On Self-Test) codes as the motherboard went through the boot process. The codes changed as each hardware component was tested, and then stopped on a specific code when a fault was detected. By checking that code against the BIOS POST code list, the technician identified the faulty hardware (such as RAM or the motherboard) that was preventing the computer from booting.

Download and install any free software diagnostic tool (such as HWMonitor, Speccy, or CrystalDiskInfo), run it on your laptop or desktop, and take a screenshot of the main dashboard showing system health details.<br><br><em><strong>Constraint:</strong> Blur or crop out any personal information before submitting.</em>

Ans :

Option 1: HWMonitor (Recommended)

- Download HWMonitor from its official website.

- Install and open the program.

- Wait a few seconds for it to detect your hardware.

- Take a screenshot of the main dashboard showing:

  - CPU temperature

  - CPU usage

  - Voltages

  - Fan speeds

  - Storage temperatures (if available)

- Blur or crop out any personal information (such as your computer name or username) before submitting.

Option 2: Speccy

- Open Speccy after installation.

- Wait for the system scan to finish.

- Take a screenshot of the Summary page.

- Blur or crop any personal information.

Option 3: CrystalDiskInfo

- Open CrystalDiskInfo.

- Take a screenshot showing:

  - Disk Health Status

  - Temperature

  - Power-on Hours

  - Drive Model (optional to blur if preferred)

Compare the use of a multimeter, POST card, and software diagnostics by creating a table with three columns (Tool, What it checks, Example Problem Detected) and fill in at least one example for each tool.

Ans :

| Tool | What it checks | Example Problem Detected |
| --- | --- | --- |
| Multimeter | Measures voltage, current, resistance, and continuity of electrical components. | Faulty laptop charger, dead CMOS battery, or no power from the computer power supply. |
| POST Card | Displays POST (Power-On Self-Test) error codes from the motherboard during startup. | Faulty RAM, motherboard, or CPU preventing the computer from booting. |
| Software Diagnostics | Monitors system health, hardware status, temperatures, storage health, and performance. | Overheating CPU, failing hard drive/SSD, low RAM, or abnormal system temperatures. |