Day – 3 Assi

Log in to your home Wi-Fi router's admin panel using its default IP address (usually 192.168.0.1 or 192.168.1.1) and take a screenshot of the login page.<br><br><em><strong>Hint:</strong> If you don't know your router's IP, open Command Prompt and run 'ipconfig' — look for 'Default Gateway'.</em>

Ans ;

Steps

- Press Win + R, type cmd, and press Enter.

- In Command Prompt, type:

ipconfig

- Find the Default Gateway under your active network adapter.

  - It is usually something like:

    - 192.168.0.1

    - 192.168.1.1

    - 10.0.0.1

- Open Google Chrome, Edge, or any browser.

- In the address bar, type the Default Gateway IP (for example, http://192.168.1.1) and press Enter.

- The router's login page should appear.

On Windows Laptop

- Press Win + R, type cmd, and press Enter.

- Type:

ipconfig

- Look under your Wireless LAN adapter Wi-Fi section. Note these three values:

| Information | Example |
| --- | --- |
| IPv4 Address | 192.168.1.105 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.1 |

It will look similar to:

Wireless LAN adapter Wi-Fi:

IPv4 Address. . . . . . . . . . : 192.168.1.105

Subnet Mask . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . : 192.168.1.1

On Android

- Open Settings → Wi-Fi.

- Tap your connected Wi-Fi network.

- Look for IP Address, Gateway, and Subnet Mask (or Network Prefix Length).

On iPhone

- Open Settings → Wi-Fi.

- Tap the ⓘ next to your connected Wi-Fi.

- Note the IP Address, Subnet Mask, and Router (this is the default gateway).

Change your device's IP address from 'Obtain IP automatically' (DHCP) to a static IP in the same range as your router (for example, set it to 192.168.1.50 if your router is 192.168.1.1), and test if you still have internet access.<br><br><em><strong>Constraint:</strong> Remember to revert to automatic/DHCP after testing to avoid network issues.</em>

Ans :

Before You Start

First, note your current network details using:

ipconfig

Suppose your network shows:

- Router (Default Gateway): 192.168.1.1

- Subnet Mask: 255.255.255.0

You can choose a static IP such as 192.168.1.50 (provided no other device is already using it).

Change to a Static IP (Windows 10/11)

- Open Settings → Network & Internet.

- Click Wi-Fi.

- Select your connected Wi-Fi network.

- Under IP assignment, click Edit.

- Change from Automatic (DHCP) to Manual.

- Enable IPv4.

- Enter:

  - IP address: 192.168.1.50

  - Subnet mask: 255.255.255.0

  - Gateway: 192.168.1.1

  - Preferred DNS: 8.8.8.8

  - Alternate DNS: 8.8.4.4

- Click Save.

Test Internet Access

Open Command Prompt and run:

ping 8.8.8.8

Then test DNS resolution:

ping google.com

Or simply open a website in your browser.

Revert to DHCP (Important)

After testing:

- Go back to Settings → Network & Internet → Wi-Fi.

- Open your connected network.

- Under IP assignment, click Edit.

- Change it back to Automatic (DHCP).

- Save the changes.

In your router's admin panel, locate the section where you can view or change the DHCP range. Write down the current range, and explain in 2-3 lines what would happen if two devices are assigned the same IP address in this range.

Ans:

Steps to Find the DHCP Range

- Log in to your router's admin page (for example, 192.168.0.1 or 192.168.1.1).

- Go to LAN, Network, or DHCP Server settings (the exact menu depends on your router's brand).

- Look for:

  - DHCP Start IP

  - DHCP End IP

Example

If your router shows:

- DHCP Start IP: 192.168.1.100

- DHCP End IP: 192.168.1.200